Lab: Using GA & MOEA/D to solve communication engineering problems

Problem description:

We have a communication system that needs to be optimized. Consider five transmitter–receiver pairs sharing one frequency channel; each transmitter’s power improves its own desired signal but may increase interference at other receivers.

We will use the public Stanford five-link power-control model (Stanford power-control example). It is a realistic communication-system model with illustrative numerical data, rather than a measured field data set. Two tasks are as follows.

Task 1 (50’):

There is a GA toolbox (GA Toolbox) that includes various GA operators as functions. Your task is to understand them and use these operator functions to build a functional GA. Then, use your GA to solve the following constrained optimization problem for communication power control. You should not use MATLAB built-in ga or gamultiobj.

The goal of the communication power control is to minimize the total transmitter power:

minimize   f(P) = P1 + P2 + P3 + P4 + P5

subject to SINRi(P) ≥ 2, i = 1, …, 5;

The search range is 0.1 ≤ Pi ≤ 5.

The five-link model and the public CVX geometric-programming example can be viewed at: Stanford power-control example. The Stanford example is provided as background and an independent reference only. Do not use, adapt, or submit its CVX geometric-programming solver as your GA solution.

The supplied starter package contains power_objective.m and power_constraints.m. The first function returns the total power for a candidate solution. The second function calculates the five SINR values and returns the nonlinear constraint values ci(P) = 1/SINRi(P) − 1/SINRspec, where SINRspec = 2. A candidate solution is feasible when ci(P) ≤ 0 for all five links; a positive constraint value indicates a violation. The optional third output, sinrValue, contains the five actual SINR values.

You are required to carry out vector programming (i.e., the population as a whole should be considered as the input). Hence, the supplied single-candidate functions need to be modified so that, for an N-by-5 population, the objective output is N-by-1, the nonlinear-constraint output is N-by-5, and the optional SINR output is N-by-5.

In your report, please show the performance of your algorithm using 20 runs (not 20 generations) using the following guidelines. In each run, use the same function evaluation budget (at most 50,000 evaluations). Save the best-so-far objective value in each generation of each run and plot the median convergence trend over the 20 runs. Report the best, worst, mean, median, and standard deviation of the final feasible objective function values in a table. For example, if in 20 runs, 18 obtained feasible solutions, the best, worst, mean, median, and standard deviation should only consider the 18 feasible runs. Also report the feasible run rate (e.g., 18/20) and the median number of evaluations needed to obtain the first feasible solution over the 20 runs.

In your report, please explain how you select the GA parameters (e.g., population size, crossover rate, mutation rate, and penalty coefficient or parameters in other constraint handling methods).

Stanford reports a minimum total power of approximately 17.0014 at P ≈ [3.6601, 3.1623, 2.9867, 4.1647, 3.0276]. You may use this as a reference.

Marking criteria for Task 1 (50 marks):

The program should obtain feasible results and demonstrate a clear convergence trend. The plot and statistical table should correctly present the required 20-run evidence.

| Criterion | Description | Partial Score | Maximum Marks |
| --- | --- | --- | --- |
| The objective/constraint functions | Correct N-by-5 vectorized evaluators and required outputs. | 0–10 | 10 |
| Result | Your GA cannot run. | 0–6 | 20 |
|  | Your GA can run, but no feasible solution is found (maximum constraint violation among the five> 1e-4). | 7–12 |  |
|  | Feasible, but 20-run evidence is incomplete or ¦median power − 17.0014¦ ≥ 0.10. | 13–17 |  |
|  | Complete 20-run evidence and ¦median power − 17.0014¦ < 0.10. | 18–20 |  |
| Plot | Correctly labeled median convergence plot from 20 runs. | 0–5 | 5 |
| Table | The required statistics are complete and correct. | 0–5 | 5 |
| Explanation | Justified GA parameters and parameters involved in constraint handling or equivalent. | 0–10 | 10 |

Task 2 (50’):

In this task, you are asked to use MOEA/D for a bi-objective optimization of the power control problem. The two objective functions to be minimized include f1(P) = ΣPi and f2(P) = −Σlog2(1 + SINRi(P)). The constraints stay the same as in Task 1.

Please propose a constraint handling method, implement MOEA/D, decide the hyperparameters, etc.

Please provide the obtained Pareto set and Pareto front values saved in a mat file. The Pareto set contains only feasible solutions that are nondominated. A solution is feasible if all five power values lie within [0.1, 5] and all five SINR values are at least 2. For example, if 200 feasible nondominated solutions remain, save them in a 200 × 5 matrix for the Pareto set and a 200 × 2 matrix for the Pareto front.

In your report, please provide the following: (1) Description of the algorithm: (i) your MOEA/D code with comments for each major function, (ii) the constraint handling method, and (iii) the MOEA/D algorithm parameter selection. (2) The Pareto front plot: Plot f1 on the x-axis and f2 on the y-axis. (3) The hypervolume value (please study the concept yourself) in the minimization space [f1, f2] using the reference point r = [26, 0]. If your run does not obtain a feasible solution, then the hypervolume value is 0.

You should perform three independent runs and select the Pareto front of the run with the median HV as the typical run.

Marking criteria for Task 2 (50 marks):

| Criterion | Description | Partial Score | Maximum Marks |
| --- | --- | --- | --- |
| MOEA/D implementation | Correct implementation of decomposition, weight vectors, neighborhoods, evolutionary operations, replacement, etc. Built-in multi-objective optimization solver (e.g., gamultiobj) is not allowed. | 0–12 | 12 |
| Constraint handling | Correct handling of the power bounds and SINR constraints. Only feasible solutions are retained in the final Pareto set. | 0–8 | 8 |
| Hyperparameter selection | The main MOEA/D parameters are clearly reported and reasonably justified. | 0–5 | 5 |
| Report quality | Completeness of the required report and correctness of the reported results. | 0–10 | 10 |
| HV performance (HV ratio = student HV / reference-PF HV; the reference-PF HV is not open to the public) | No valid feasible Pareto set or invalid HV calculation. | 0–3 | 15 |
|  | 0 < HV ratio < 0.50. | 4–8 |  |
|  | 0.50 ≤ HV ratio < 0.75. | 9–11 |  |
|  | 0.75 ≤ HV ratio < 0.90. | 12-13 |  |
|  | HV ratio ≥ 0.90. | 14-15 |  |

Submission: Please submit your modified vectorized evaluator files, Task 1 GA code, Task 2 MOEA/D code, the required mat files, and a PDF report. Please do not submit unmodified toolbox files.